# NET512

# Designing and Implementing a Secure, Scalable, and Automated

# Enterprise Network

# Weekly Practical Plan — NQF Level 5 | 17 Credits

Week 3: Life of a Packet (10 Marks)

| R1 | R2 | R3 | DNS-Server |
| --- | --- | --- | --- |
| 10.10.10.1 | 10.10.10.2 / 10.10.20.2 | 10.10.20.1 | 10.10.10.10 |
| 255.255.255.0 | 255.255.255.0 | 255.255.255.0 | 255.255.255.0 |

# DNS Configuration

## 16. Configure the routers as DNS clients, pointing to the DNS server in the topology.

## 17. Test name resolution from a router by pinging a hostname instead of an IP address.

# ARP Cache

18. Clear the ARP cache on a router, then generate traffic (e.g., a ping) to a directly connected neighbour.

## 19. Examine the ARP cache on the routers with 'show arp' and identify the newly learned entry.

# Reflection Questions (answer these in your submission):

## 20. What information does an ARP entry give you that a routing table entry does not?

An ARP entry provides the MAC address associated with an IP address, as well as the interface used to reach that device. A routing table provides information about where to forward packets but does not provide the device's MAC address.

## 21.Why does a router need to perform an ARP lookup even after it already knows the correct route from its

## routing table?

The routing table tells the router the correct next hop or outgoing interface, but the router still needs the next-hop MAC address to create the Ethernet frame. ARP is used to find that MAC address so the packet can be de livered on the local network.